Electronics Documentation

### 1. Electronics Block Diagram

Signal and  power flow

### 2. Components Table

| # Component | # Quantity | # Purpose |  | # Justification |
| --- | --- | --- | --- | --- |
| # ESP32 Microcontroller | # 1 | # Acts as the central brain, processing sensor inputs and orchestrating all mechanical and visual outputs. |  | # Offers ample GPIO pins with hardware PWM support needed to drive multiple servos simultaneously. It also provides the processing speed required to constantly poll sensors without lagging the motor routines. |
| # Servo Motors | # 5 | # Actuates the physical movements of the robot's left eye, right eye, left eyelid, right eyelid, and head. |  | # Servos provide precise, repeatable angular positioning out of the box. This is essential for giving the robot lifelike, expressive movements without the need to build complex custom feedback loops. |
| # PIR Motion Sensor | # 1 | # Detects the presence of a person in front of the robot to trigger the interactive "wake" sequence. |  | # Highly reliable for human body heat detection over a wide angle. It acts as an ideal, low-power trigger to pull the robot out of its resting state. |
| # Sound Sensor Module | # 1 | # Listens for sharp noises while the robot is active to trigger the secondary head-turning routine. |  | # Provides a simple binary signal when a volume threshold is crossed, allowing for easy integration into the ESP32's polling loop without requiring heavy audio processing. |
| # Neo Pixel LED Array(8 LEDs) | # 1 | # Serves as a dynamic visual status indicator, pulsing green during head movements and turning red after prolonged inactivity. |  | # Requires only a single data pin from the microcontroller while offering individual, full-color RGB control, maximizing visual impact while minimizing wiring clutter. |
| # 5V External Power Supply (Min. 3A) | # 1 | # Delivers the necessary electrical current to run the microcontroller, sensors, LEDs, and all five motors simultaneously. |  | # Five servos moving at once will easily overwhelm a standard USB connection or the ESP32's onboard voltage regulator, causing immediate brownouts. A dedicated power source ensures system stability. |